# README file

Dataset Title: Raw experimental data for: "Cannabinoid monitoring in advanced dried blood microsamples: a validated HPLC-MS/MS multianalyte method" — extraction solvent screening, extraction yield, matrix effect and stability

Dataset Author/s:

Michele Protti (Research Group of Pharmaco-Toxicological Analysis (PTA Lab), Department of Pharmacy and Biotechnology (FaBiT), Alma Mater Studiorum - University of Bologna);

Laura Mercolini (Research Group of Pharmaco-Toxicological Analysis (PTA Lab), Department of Pharmacy and Biotechnology (FaBiT), Alma Mater Studiorum - University of Bologna);

Roberto Mandrioli (Research Group of Advanced Pharmaceutical Analysis (APA Lab), Department for Life Quality Studies (QuVi), Alma Mater Studiorum – University of Bologna);

Data Set Contact Person/s:

Michele Protti (Research Group of Pharmaco-Toxicological Analysis (PTA Lab), Department of Pharmacy and Biotechnology (FaBiT), Alma Mater Studiorum - University of Bologna);

Data Set License:

This data set is distributed under a Creative Commons Attribution 4.0 International (CC BY 4.0)

Publication Year: 2026

Project Info:

ESILARANTE (ElectroSpray Immobilization of LAccase foR cANnabinoids deTEction), Project ID: 202283YHXY. Funded by the Italian Ministry of University and Research under the National Recovery and Resilience Plan (PNRR), Mission 4, Component 2, Investment 1.1 – PRIN 2022, funded by the European Union – NextGenerationEU.

## Data set Contents

Files in this dataset:

Research_Raw_Dataset_Cannabinoid_cDBS.xlsx (98.42 KB)
README_Cannabinoid_cDBS.docx

The dataset consists of one spreadsheet named Research_Raw_Dataset_Cannabinoid_cDBS.xlsx containing four sheets:

- README. Contains documentation for the dataset.

- Solvent_Screening. EY%, ME%, IS-ME% for 7 solvent conditions (screening phase, 200 µL per disc). 6 donors × LQC+HQC × 9 analytes. Underlying data for solvent selection (Section 3.1.2).

- EY_ME_Optimised. EY%, ME%, IS-ME% for the final validated extraction protocol (100 µL FA/MeOH). 6 donors × LQC+HQC × 9 analytes. Underlying data for Table 3 (EY% and IS-ME% columns).

- Stability. Measured concentrations and % deviation from freshly prepared reference under all stability conditions (8 conditions × LQC+HQC × 6 replicates × 9 analytes). Underlying data for Section 3.2.5.

## Abstract

This dataset provides raw chromatographic peak area data underlying the extraction solvent screening, final-method extraction yield and matrix effect assessment, and stability experiments described in the associated manuscript. Data are presented as three spreadsheet tables; derived parameters (EY%, ME%, IS-ME%, % deviation from reference) are included per row to facilitate independent verification of the reported validation outcomes.

## List of acronyms

11-OH-THC	11-hydroxy-Δ9-tetrahydrocannabinol

6α-OH-CBD	6α-hydroxy-cannabidiol

7-COOH-CBD	7-carboxy-cannabidiol

7-OH-CBD	7-hydroxy-cannabidiol

CBD	Cannabidiol

CBG	Cannabigerol

CBN	Cannabinol

cDBS	Capillary dried blood spot

ESI+	Electrospray ionisation in positive mode

ESI−	Electrospray ionisation in negative mode

EY%	Extraction yield (%)

FA/MeOH	Formic acid in methanol

hemaPEN	Volumetric dried blood microsampling device (Trajan Scientific and Medical, Melbourne, Australia)

HQC	High-concentration quality control sample

HPLC	High-performance liquid chromatography

IS	Internal standard

IS-ME%	IS-normalised matrix effect (%)

LLOQ	Lower limit of quantification

LQC	Low-concentration quality control sample

ME%	Matrix effect (%)

MQC	Medium-concentration quality control sample

MRM	Multiple reaction monitoring

MS	Mass spectrometry

MS/MS	Tandem mass spectrometry

QC	Quality control

THC	Δ9-tetrahydrocannabinol

THC-COOH	11-nor-9-carboxy-Δ9-tetrahydrocannabinol

THC-d3	Δ9-tetrahydrocannabinol-d3 (deuterated internal standard)

UAE	Ultrasonic-assisted extraction

ULOQ	Upper limit of quantification

## Methodology

Whole blood samples were collected by minimally invasive fingerstick using the Winnoz Haiim® capillary blood collection device and subsequently spotted onto hemaPEN volumetric microsampling cartridges (Trajan Scientific and Medical, Melbourne, Australia). Each hemaPEN cartridge integrates four microcapillaries, each collecting 2.74 µL of blood by capillary action onto a prepunched PE 226 filter paper disc (3.5 mm diameter).

Analytes were extracted by UAE in FA/MeOH 0.1% (V/V) at 25 °C for 30 min, followed by evaporation under nitrogen and reconstitution in 20:80 (V/V) 0.1% formic acid in acetonitrile:water. Chromatographic analysis was performed on a Waters Alliance e2695 HPLC system coupled to a Micromass Quattro Micro triple quadrupole mass spectrometer (Waters, Manchester, UK), with ESI in both positive and negative mode and polarity switching. Separation was achieved on a SunShell C8 column (100 × 2.1 mm, 2.6 µm; ChromaNik Technologies, Japan). Detection was in MRM mode. Data were acquired and processed with MassLynx 4.1 software (Waters).

Extraction yield and matrix effect were assessed using the Matuszewski three-set design. Stability was assessed under the following conditions: room-temperature storage (30, 60, 90 days); accelerated thermal stress (40 °C, 7 days); freeze-thaw cycling (three cycles, −20 °C ↔ room temperature); processed extract stability (24 h at 10 °C in the autosampler); and pre-spotting stability in liquid capillary blood (2 h and 4 h at room temperature).

## Notes

The nine target analytes are, in the analytical order used throughout the dataset: CBD, 7-OH-CBD, 7-COOH-CBD, 6α-OH-CBD, CBN, CBG, THC, 11-OH-THC and THC-COOH. Eight deuterium-labelled internal standards were used: CBD-d3 (IS4), 7-OH-CBD-d3 (IS5, also surrogate IS for 6α-OH-CBD), 7-COOH-CBD-d3 (IS6), CBN-d3 (IS7), CBG-d3 (IS8), THC-d3 (IS1), 11-OH-THC-d3 (IS2) and THC-COOH-d3 (IS3).