# README file

Dataset Title: “In-Vitro Tibiofemoral Kinematics Dataset Under External Loading: Experimental Dataset for a Continuous Non-Linear Map of the Knee Response to Load”

Dataset Authors:

- Lorenzo Capetta (Department of Industrial Engineering (DIN), Alma Mater Studiorum - University of Bologna, Bologna, Italy), https://orcid.org/0009-0007-6625-8118;

- Nicola Sancisi (Department of Industrial Engineering (DIN), Alma Mater Studiorum - University of Bologna, Bologna, Italy), https://orcid.org/0000-0003-3005-6489;

- Michele Conconi (Department of Industrial Engineering (DIN), Alma Mater Studiorum - University of Bologna, Bologna, Italy), https://orcid.org/0000-0003-2392-2684;

Data Set Contact Person:

- Lorenzo Capetta (Department of Industrial Engineering (DIN), Alma Mater Studiorum - University of Bologna, Bologna, Italy), https://orcid.org/0009-0007-6625-8118, lorenzo.capetta@unibo.it;

Data Set License:

This data set is distributed under a Creative Commons Attribution 4.0 International (CC BY 4.0), https://creativecommons.org/licenses/by/4.0/.

Publication Year: 2026

## Data set Contents

The dataset consists of 1 compressed archive, named “KRLM_dataset_IT-BO_20260923_archive.zip” containing the full dataset structure:

- 6 “Leg” folders, one for each specimen

Leg01, …, Leg06

  - Inside each “Leg” folder, text files containing data from femoral kinematics acquisitions under different motion settings and loading conditions

e.g. “KRLM_Leg01_IT-BO_20250428_LHR-Fx_n_050.txt”

e.g. “KRLM_Leg03_IT-BO_20250428_LHR-Fy_n_150_My_p_010.txt”

e.g. “KRLM_Leg04_IT-BO_20240220_PHR-passive_rep04.txt”

- 1 README file

“KRLM_dataset_IT-BO_20260923_README.docx”

## Abstract

This dataset contains the experimental measurements used to derive and validate a continuous non-linear map of the human knee response to load. Data were collected in vitro from six fresh-frozen cadaveric lower limbs obtained from five donors, using a custom cable-driven loading rig and a stereophotogrammetric acquisition system. For each specimen, tibiofemoral kinematics were recorded continuously across the full flexion range under unloaded conditions and under multiple monoaxial loading cases, including forces along the anterior-posterior, proximal-distal, and medial-lateral directions and torques about the abduction-adduction and internal-external axes. Additional combined-loading experiments are included for validation. The dataset supports the reconstruction of natural knee motion and load-induced deformations as functions of flexion angle, load magnitude, and load direction, and can be used for kinematic load-dependent mapping, multibody and musculoskeletal modelling, model calibration, and validation studies in joint biomechanics. This dataset updates the previous contribution, “In-Vitro Tibiofemoral Kinematics Dataset Under External Loading: Experimental Dataset for a Continuous Nonlinear Knee Compliance Map”, available in the AMSActa archive. The changes are limited to notation updates and the addition of references to provide further clarification on the experimental apparatus and setup.

## Content of the files:

The dataset contains text files with processed femoral kinematics acquired from six cadaveric lower limbs under different motion and loading conditions. Each .txt file contains the kinematic trajectory of the femur with respect to the tibia after preprocessing and realignment. The specific variables, units, coordinate conventions, and column meanings are found below in this README. Files are organised by leg, motion set, and loading condition. Here some examples:

- file “KRLM_dataset_IT-BO_20260923_README.docx” contains the description of the dataset structure, the adopted file naming convention, the meaning of abbreviations and codes, and general information needed to interpret the deposited files.

- folder “KRLM_Leg01” contains all data associated with Leg 1. Analogous folders are provided for data related to legs 2 to 6 (“Leg02” to “Leg06”).

- file “KRLM_Leg01_IT-BO_20250428_LHR-Fx_n_050.txt” contains the processed femoral kinematics for Leg 1, collected in Bologna (Italy) on 2025-04-28 (“IT-BO_20250428”), for the loaded, healthy, and realigned motion set (“LHR”) under monoaxial loading condition (a negative force applied to the tibia along the Fx direction with magnitude 50 N, “Fx_n_050”).

- file “KRLM_Leg03_IT-BO_20250428_LHR-Fy_n_150_My_p_010.txt” contains the processed femoral kinematics for Leg 3 (“Leg03”), collected in Bologna (Italy) on 2025-04-28 (“IT-BO_20250428”), for the loaded, healthy, and realigned motion set (“LHR”) under a combined loading condition (a negative Fy force of magnitude 150 N and a positive My torque of magnitude 10 Nm applied to the tibia, “Fy_n_150_My_p_010”).

- file “KRLM_Leg04_IT-BO_20240220_PHR-passive_rep04.txt” contains the processed femoral kinematics for Leg 4 (“Leg04”), collected in Bologna (Italy) on 2024-02-20 (“IT-BO_20240220”) in a passive motion acquisition (“PHR”).

## File specifications

The dataset consists mainly of plain-text (.txt) files containing processed femoral kinematics for six legs, two motion sets, and multiple loading conditions. These files are encoded in UTF-8 and store numeric data in tab-delimited format, with dot as decimal separator. A documentation file in .docx format is also provided to describe the dataset structure, naming convention, abbreviations, and metadata.

Text data files

- Format: .txt

- Content: processed femoral kinematic data for one leg, one motion set, and one loading condition

- Character encoding: UTF-8

- Field delimiter: tab

- Text delimiter: none

- Decimal separator: dot (.)

Each .txt file contains a numeric table and can be imported directly into common numerical environments. If needed, files can be converted to .csv format using MATLAB, Python, LibreOffice Calc, or spreadsheet software.

Documentation file

- Format: .docx

- Content: human-readable documentation describing dataset organisation, file naming rules, abbreviations, and metadata

Compressed archive

If the dataset is deposited as a compressed package:

- Preferred format: .zip

- Content: complete dataset folder structure, including data files and documentation

Notes on reuse

- Filenames were standardised to make each file self-descriptive and to include the main metadata directly in the filename.

- Data files were kept in plain text format to maximise long-term accessibility and interoperability.

- The README file should be consulted before reuse, as it explains the meaning of folder names, abbreviations, load-condition codes, and leg identifiers.

## Methodology

The dataset contains processed tibiofemoral kinematics acquired in vitro from six fresh-frozen lower-limb specimens from five donors, mounted on a force-controlled, cable-driven custom loading rig. Knee motion was measured using a stereophotogrammetric system (Vicon Motion Systems, Ltd., Oxford, UK), comprising 6 Bonita and 2 Vero cameras. Further information about the loading rig and the stereophotogrammetric acquisition system can be found at (Forlani, Sancisi, Conconi, & Parenti-Castelli, 2016) and (Goldfarb, Lewis, Tacescu, & Fischer, 2021), respectively. Cameras were positioned to minimise occlusion artifacts, according to (Conconi, Pompili, Sancisi, & Parenti-Castelli, 2021).

After leg skeletonisation, the trackers were rigidly fixed directly to the exposed femoral and tibial shafts, respectively, using bone screws, thus introducing no soft-tissue artefacts. The trackers were recorded during continuous flexion up to approximately 140°. For each leg, an unloaded condition was first acquired under a baseline compressive load of 50 N to estimate the natural knee motion.

For each leg, one baseline unloaded trial was acquired to represent natural knee motion.  Additional acquisitions were then performed under monoaxial external loads, including forces along the anterior-posterior (AP), proximal-distal (PD), and medial-lateral (ML) directions, and torques about the abduction-adduction (AA) and internal-external (IE) axes. Legs 1 to 3 also include combined-loading conditions used for validation. Folder and file names identify the leg, motion set, acquisition date, and loading condition.

Before deposition, the data were processed and realigned. Translational components were scaled to account for inter-subject dimensional differences, using femur volume as scaling reference. A registration procedure was then applied to reduce variability related to anatomical reference-system definition, by aligning the natural motion of each leg to that of a selected reference specimen. The same registration was subsequently applied to the corresponding loaded trials.

## List of variables

The deposited .txt files contain processed femoral kinematics with respect to the tibia. The 6 motion components are stored in the files in ordered, separated columns as follows:

- FE: flexion-extension angle, in degrees (deg)

- AA: abduction-adduction angle, in degrees (deg)

- IE: internal-external rotation angle, in degrees (deg)

- AP: anterior-posterior translation, in millimetres (mm)

- PD: proximal-distal translation, in millimetres (mm)

- ML: medial-lateral translation, in millimetres (mm)

The motion setting associated with each file is encoded in the filename rather than stored as a column variable. The following abbreviations are used:

- LHR: loaded, healthy, and realigned motion set

- PHR: passive, healthy, and realigned motion set

The loading condition associated with each file is encoded in the filename rather than stored as a column variable. The following abbreviations are used:

- Fx, Fy, Fz: external force components applied to the tibia along the anatomical axes

- Mx, My: external torque components applied to the tibia about the anatomical axes

- p, n: positive and negative loading direction

- Numerical values following load codes indicate the load magnitude

  - forces in newtons (N)

  - torques in newton-metres (Nm)

e.g. In the filename:

KRLM_Leg01_IT-BO_20250428_LHR-Fy_n_150_My_p_010.txt

The encoded loading condition indicates:

- Fy_n_150: negative force along the Fy direction, magnitude 150 N

- My_p_010: positive torque about the My axis, magnitude 10 Nm

## Notes

- The dataset contains processed and realigned kinematic data, not raw motion-capture or raw tracker data.

- Some motion sets may contain repeated acquisitions, identified by the suffix “repXX”.

- The naming convention was standardised to make each file self-descriptive and consistent across the whole dataset.

- Loading conditions in this dataset are expressed as external forces applied to the tibia rather than to the femur. By action-reaction equivalence, the corresponding force acting on the femur has identical magnitude and opposite sign. Thus, the deformation associated with a positive 50 N force applied to the femur is represented here by a negative 50 N force applied to the tibia.

- Cited references in the script point, respectively, to:

  - Conconi, M., Pompili, A., Sancisi, N., & Parenti-Castelli, V. (2021). Quantification Of The Errors Associated With Marker Occlusion In Stereophotogrammetric Systems And Implications On Gait Analysis. Journal of Biomechanics, 114, 110162.

  - Forlani, M., Sancisi, N., Conconi, M., & Parenti-Castelli, V. (2016). A new test rig for static and dynamic evaluation of knee motion based on a cable-driven parallel manipulator loading system. Meccanica, 1571-1581.

  - Goldfarb, N., Lewis, A., Tacescu, A., & Fischer, G. S. (2021). Open Source Vicon Toolkit For Motion Capture And Gait Analysis. Computer Methods and Programs in Biomedicine, 212, 106414.